6.4 Thermal Information: OPA227, OPA228

| THERMAL METRIC(1) |  | OPA227P, OPA227PA, OPA228P, OPA228PA | OPA227U, OPA227UA, OPA228U, OPA228UA | UNIT |
| --- | --- | --- | --- | --- |
|  |  | P (PDIP) | D (SOIC) |  |
|  |  | 8 PINS | 8 PINS |  |
| RθJA | Junction-to-ambient thermal resistance | 48.9 | 110.1 | °C/W |
| RθJC(top) | Junction-to-case(top) thermal resistance | 37.7 | 52.2 | °C/W |
| RθJB | Junction-to-board thermal resistance | 26.1 | 52.3 | °C/W |
| ψJT | Junction-to-top characterization parameter | 15.1 | 10.4 | °C/W |
| ψJB | Junction-to-board characterization parameter | 26 | 51.5 | °C/W |
| RθJC(bot) | Junction-to-case(bottom) thermal resistance | N/A | N/A | °C/W |

(1) For more information about traditional and new thermal metrics, see the Semiconductor and IC Package Thermal Metrics application report.

6.5 Thermal Information: OPA2227, OPA2228

| THERMAL METRIC(1) |  | OPA2227P, OPA2227PA, OPA2228P, OPA2228PA | OPA2227U, OPA2227UA, OPA2228U, OPA2228UA | UNIT |
| --- | --- | --- | --- | --- |
|  |  | P (PDIP) | D (SOIC) |  |
|  |  | 8 PINS | 8 PINS |  |
| RθJA | Junction-to-ambient thermal resistance | 110.1 | 101.9 | °C/W |
| RθJC(top) | Junction-to-case(top) thermal resistance | 52.2 | 46.3 | °C/W |
| RθJB | Junction-to-board thermal resistance | 52.3 | 45.5 | °C/W |
| ψJT | Junction-to-top characterization parameter | 10.4 | 6.6 | °C/W |
| ψJB | Junction-to-board characterization parameter | 51.5 | 42.8 | °C/W |
| RθJC(bot) | Junction-to-case(bottom) thermal resistance | N/A | N/A | °C/W |

(1) For more information about traditional and new thermal metrics, see the Semiconductor and IC Package Thermal Metrics application report.

6.6 Thermal Information: OPA4227, OPA4228

| THERMAL METRIC(1) |  | OPA4227P, OPA4227PA, OPA4228P, OPA4228PA | OPA4227U, OPA4227UA, OPA4228U, OPA4228UA | UNIT |
| --- | --- | --- | --- | --- |
|  |  | N (PDIP) | D (SOIC) |  |
|  |  | 14 PINS | 14 PINS |  |
| RθJA | Junction-to-ambient thermal resistance | 65.5 | 65 | °C/W |
| RθJC(top) | Junction-to-case(top) thermal resistance | 20 | 23.1 | °C/W |
| RθJB | Junction-to-board thermal resistance | 25.9 | 20.3 | °C/W |
| ψJT | Junction-to-top characterization parameter | 1.9 | 1.8 | °C/W |
| ψJB | Junction-to-board characterization parameter | 25.3 | 19.9 | °C/W |
| RθJC(bot) | Junction-to-case(bottom) thermal resistance | N/A | N/A | °C/W |

(1) For more information about traditional and new thermal metrics, see the Semiconductor and IC Package Thermal Metrics application report.