6.7 Electrical Characteristics: OPAx227

at TA = +25°C, VS = ±5 V to ±15 V, VCM = VOUT = VS / 2, and RL = 10 kΩ connected to VS / 2 (unless otherwise noted)

| PARAMETER |  | TEST CONDITIONS |  | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OFFSET VOLTAGE |  |  |  |  |  |  |  |
| VOS | Input offset voltage | U, P grades |  | ±5 |  | ±75 | μV |
|  |  |  | TA=-40°C to +85°C | ±100 |  |  |  |
|  |  | UA, PA grades |  | ±10 |  | ±200 |  |
|  |  |  | TA=-40°C to +85°C | ±200 |  |  |  |
| dVOS/dT | Input offset voltage drift | U, P grades | TA=-40°C to +85°C | ±0.1 |  | ±0.6 | μV/°C |
|  |  | UA, PA grades | TA=-40°C to +85°C | ±0.3 |  | ±2 |  |
| PSRR | Power-supply rejection ratio | VS=±2.5 V to ±18 V |  | ±0.5 |  | ±2 | μV/V |
|  |  |  | TA=-40°C to +85°C | ±2 |  |  |  |
|  | Long-term drift |  |  | 0.2 |  |  | μV/mo |
|  | Channel separation (dual, quad) | DC |  | 0.2 |  |  | μV/V |
|  |  | f = 1 kHz, RL=5 kΩ |  | 110 |  |  | dB |
| INPUT BIAS CURRENT |  |  |  |  |  |  |  |
| IB | Input bias current |  |  | ±2.5 |  | ±10 | nA |
|  |  | TA=-40°C to +85°C |  | ±10 |  |  |  |
| IOS | Input offset current |  |  | ±2.5 |  | ±10 | nA |
|  |  | TA=-40°C to +85°C |  | ±10 |  |  |  |
| NOISE |  |  |  |  |  |  |  |
|  | Input voltage noise | f = 0.1 Hz to 10 Hz |  | 90 |  |  | nVPP |
|  |  |  |  | 15 |  |  | nVRMS |
| en | Input voltage noise density | f = 10 Hz |  | 3.5 |  |  | nV/√Hz |
|  |  | f = 100 Hz |  | 3 |  |  |  |
|  |  | f = 1 kHz |  | 3 |  |  |  |
| in | Input current noise density | f = 1 kHz |  | 0.4 |  |  | pA/√Hz |
| INPUT VOLTAGE |  |  |  |  |  |  |  |
| VCM | Common-mode voltage |  |  | (V-)+2 | (V+)-2 |  | V |
| CMRR | Common-mode rejection ratio | (V-)+2 V < VCM <(V+)-2 V |  | 120 | 138 |  | dB |
|  |  |  | TA=-40°C to +85°C | 120 |  |  |  |
| INPUT IMPEDANCE |  |  |  |  |  |  |  |
|  | Differential |  |  | 10¦¦12 |  |  | MΩ ¦¦ pF |
|  | Common-mode | VCM =(V-)+2 V to (V+)-2 V |  | 1¦¦3 |  |  | GΩ ¦¦ pF |